D E S C R I P T I O N  O F  S E T U P - U T I L I T Y

The   program SETUP is used for configuring the ITT 3030  printer

interface.

Setting-up of the standard interface (below the monitor cable) or

of  the  optional  interface ( SIO-card with  adapter  cable ) is

possible.

The default values  are deleted by a  cold start.  With the  warm

start "CTRL C", all values are preserved.

All current  values  are automatically stored in  a file  on  the

same diskette  as the  SETUP program. When  calling  the  program

again,  this  information is  available  as default  value ( pre-

selected value ).

In addition, the SETUP program offers the possibility of  setting

or modifying the I/O-BYTE as  follows:  the  List  Device  Output

( selection "O" in the main menu) can be assigned to

- the PIO-card (Centronics-Interface = TTY:)

- the monitor = CON:

- the standard serial interface (COMBO = LPT:)

- the SIO-card channel 1 = UL1:

The menu control has two levels -  one main menu and  eight sub-

menus. Through the main menu a specific sub-menu is called. After

selecting the desired set-up values  within  the   sub-menu,  the

program automatically returns to the main menu.

Three options are possible:

- Adoption  of  default   value  and  return  to CP/M by

pressing "ESC".

- Adoption of default value and return  to the main menu

by pressing "RETURN".

- Setting-up of a new value and return to the main menu.

The SETUP program is menu-driven, and therefore self-explanatory.

Here the main menu   and  the   eight sub-menus ( selection 0-7 )

shown in numerical order:

SETUP MAIN MENU

BACK TO CP/M :          TYPE ESCAPE

SELECT DEFAULT VALUES : TYPE <RETURN>

SET LIST DEVICE           =  0

SET COMBO BAUDRATE        =  1

SET SIO CHANNEL           =  2

SET SIO BAUDRATE          =  3

SET SIO PARITY            =  4

SET SIO STOP-BITS         =  5

SIO 7/8 BIT LENGTH        =  6

SET SIO AUTO ENABLE       =  7

=====================================================

SET LIST DEVICE                             (OPTION O)

DEFAULT = TTY

LST:= TTY:  (PIO OUTPUT)    = 1

LST:= CRT:  (CONSOLE)       = 2

LST:= LPT:  (COMBO OUTPUT)  = 3

LST:= UL1:  (SIO 1 OUTPUT)  = 4

-----------------------------------------------------

SET COMBO BAUDRATE                         (OPTION 1)

DEFAULT = 4800

4800 = 1

2400 = 2

1200 = 3

600 = 4

300 = 5

-----------------------------------------------------

SET SIO CHANNEL 1 - 4                      (OPTION 2)

DEFAULT = CHANNEL 1

ENTER CHANNEL NUMBER (1 - 4)

-----------------------------------------------------

----------------------------------------------------                 SET SIO BAUDRATE           (OPTION 3)

WARNING: BAUDRATE CH. 1 = CH. 3, CH. 2 = CH. 4

DEFAULT = 4800

19200 = 0

9600 = 1

4800 = 2

2400 = 3

1200 = 4

600 = 5

300 = 6

150 = 7

75 = 8

110 = 9

-----------------------------------------------------

SET PARITY                           (OPTION 4)

DEFAULT = NO PARITY

NO PARITY   = 0

PARITY ODD  = 1

PARITY EVEN = 2

-----------------------------------------------------

SET STOP BITS                              (OPTION 5)

DEFAULT = 1 STOP BIT

1     STOP BIT   = 1

1 1/2 STOP BITS  = 2

2     STOP BITS  = 3

-----------------------------------------------------

SET SIO 7 OR 8 BIT LENGTH                  (OPTION 6)

DEFAULT = 8 BIT

7 BIT = 0

8 BIT = 1

-----------------------------------------------------

SET AUTO ENABLE ON/OFF                     (OPTION 7)

DEFAULT = AUTO ENABLE OFF

AUTO ENABLE OFF  = 0

AUTO ENABLE ON   = 1

-----------------------------------------------------